Уважаемый Переводчик, пожалуйста, впишите свое ФИО и выполните перевод на русский язык отрывка.

| ФИО |  |
| --- | --- |
| Description/characteristics Firmly fixed press sleeve, permanently connected with the fitting body protects against mechanical damage to the O-ring Press sleeves with inspection windows, the penetration depth of the pipe into the fitting can be checked before pressing After installation the form-stable press sleeve allows the connection to absorb bending forces without developing leaks. This allows a pipe that has already been installed to be realigned after installation (up until the pressure test) Description/characteristics During installation the Uponor Unipipe MLC pipe it is pushed between the supporting sleeve and stainless steel press sleeve and a force-closed connection is made with the Uponor composite fitting. Pressed into the inner plastic layer of the pipe, the special profile of the PPSU insert produces a reliable connection. A high- temperature and age resistant O-ring fitted into a groove provides sealing between the insert part and inner wall of the pipe. After installation the form-stable press sleeve allows the connection to absorb bending forces without developing leaks. This allows a pipe that has already been installed to be realigned after installation (up until the pressure test) |  |